04/05 FORD CREW CAB

EASY REACH INSTALL INSTRUCTIONS

Right side

- Remove rt. seat assembly, and rt. lower seat belt at floor, (if equipped with center console remove it also, this will be reinstalled later).

- Set the EASY REACH unit into the truck.  If equipped with front mid seat, the Easy Reach must go below the seat rt. front pad. See picture #  . Rotate the East Reach seat swivel by pushing the RED release lever to disengage the latch, turn the platform about ¼ turn to clear rear mount holes. Use OEM seat bolts at rear of unit and supplied M12 x 1.75x 40 MM bolts at front.

- See picture # 2 where the seat skirt needs to be trimmed to clear gussets on Easy Reach base. Install the OEM truck seat on the EASY REACH use supplied M12 nuts and bolts at rear and M10- 1.5 x30 bolts and nuts on front.

- Wiring, the controller unit will be mounted under or behind the rear seat. Using supplied 10 Ga. Wiring mount the 30 amp circuit breaker as close to the battery as possible, use grommets if going through firewall. The ground circuit needs a clean connection, (not a seat bolt). Protect with an aerosol spray when completed. NOTE, the controller is polarized the red (hot) wire from battery must go to the red wire on the controller. Failure to connect properly will damage the unit and void the warranty. The orange wire from controller will go to the supplied door cutout switch. This is a normally open switch, opening the door will complete the circuit.

- Reengage the seat swivel by again latching the RED release lever. Carefully lower the seat to assure clearances at swivel areas and rocker panel areas.

- Reinstall the rt. seat belt about 5 in. to the rear of existing belt, use the rubber plugged hole already in floor and original bolt with supplied reinforcing plate and nut.

- Test unit with weight on seat to insure adequate clearances at this time.                                                                                                                                                                                                                                                                 n                                                                                                                                                      THANK YOU for purchasing at ACCESS UNLIMITED                                                                      any questions please call us at 1- 800 849 2143