Vehicle specific instructions for 2006 Dodge Dakota 4 Door Crew

- Remove front seat.  After removing bolts, tilt seat back and unplug wire connector from bottom. IMPORTANT:  Once seat wiring is unplugged, DO NOT turn key.  To move vehicle, must plug in seat.

- Remove front and rear kick plates and “B” pillar cover.

- Remove front door panel cut lower portion of panel (see picture # 1) and replace with supplied panel.

- Remove speaker, take spacer off speakers.  Mount speakers without spacer (see picture # 2 and # 3).

- Grind off rivets on seat brackets and remove brackets (see pictures # 4 and #5).

- Cut corner of out-board rear seat where sealt belt anchor point is (see picture # 6).

- Cut out-board rear seat track (see picture # 7).

- Remove computer brace from bottom of seat between tracks, flatten enough to clear seat plate and refasten.

- Place unit in vehicle and fasten to floor with original bolts.  Place bottom half of console mount on rear on-board bolt before installing (see picture # 9).

- Install rear front console brackets (see pictures # 8, # 9, # 10, #11, # 12).

- Cover “B” pillar with supplied material.

- Weld stops to front of seat tracks (see picture # 13).

- With rotate motor disengaged, rotate unit out door.  Install seat to unit using supplied material.

Vehicle specific packing list for 2006 Dodge Dakota 4 Door Crew

1 – Lower door panel for front door

1 – 16” x 30” sheet vinyl for “B” pillar

1 at front and rear console mounting brackets and hardware

1 – Seat belt bolt and washer (Grade 8)

1 – Long recline handle

2 – Seat track stops/caps (see picture # 13)

2 at 5/16 – 18 x 1” button head screws

2 at 5/16 – 18 x 1 1/2” flat head screws

2 at 5/16 – 18 nylock nuts